TriSense - simple guide for the team

TriSense is a friendly robot that:

- sees with the camera (HuskyLens),

- speaks in a friendly voice,

- recognizes a child and LEGO pieces,

- responds through a "brain" running on the laptop.

What the project does, in a nutshell

Imagine TriSense as a two-part game:

- The robot (ESP32) = the "body" (camera, sound, connection to the LEGO hub).

- The laptop (Python + trisense) = the "brain" (thinks of the answer and decides what the robot says).

When the camera sees something:

- The robot sends a message.

- The brain on the laptop understands it.

- TriSense responds with a short and positive message.

Important components

| compound | What does he do (in everyone's understanding) |
| --- | --- |
| main_robot.py | The main program on the robot (camera + audio + MQTT) |
| run_trisense_brain.py + trisense/ | The laptop program that "thinks" the answer |
| run_voice_dialog.py + trisense/voice_tcp_server.py | The variant in which the robot sends the voice to the laptop (TCP server for audio) |
| RECOVERY.md | What to do if the upload/repl doesn't work |
| HARDWARE_LMS_ESP32_ANTONS.md | Useful hardware notes for the LMS-ESP32 board |

Hardware LMS-ESP32 (Anton's Mindstorms)

Official documentation:

- Pinout: https://www.antonsmindstorms.com/docs/lms-esp32-v2-pinout/

- Getting started: https://www.antonsmindstorms.com/docs/getting-started-with-your-new-lms-esp32v2-board/

- All expansion cards: https://www.antonsmindstorms.com/doc-category/expansion-board-documentation/

Pin diagram (TriSense)

Mapping used in the current code (main_robot.py):

| Module | Signal | GPIO | Observations |
| --- | --- | --- | --- |
| LEGO Hub (LPF2/PUPRemote) | UART to hub | 7/8 | Dedicated LMS-ESP32 line for connection to the LEGO hub |
| HuskyLens (I2C) | SCL | 22 | SoftI2C(scl=Pin(22), sda=Pin(21)) |
| HuskyLens (I2C) | SDA | 21 | Default for the room in the project |
| I2S speaker (MAX98357) | BCLK | 14 | I2S audio clock (SCK on microphone) |
| I2S speaker (MAX98357) | LRC/WS | 15 | Word select I2S |
| I2S speaker (MAX98357) | FROM | 26 | Audio data to amplifier |
| Amplifier | EN | 32 | Amplifier activation |
| I2S Microphone (INMP441) | SD | 33 | Microphone data (RX), on the same SCK/BCLK and WS/LRC |

Quick diagram:

LMS-ESP32

├ ─ LEGO Hub (UART): GPIO 7/8

├ ─ HuskyLens I2C: SCL=22, SDA=21

└─ Audio

├ ─ MAX98357 (speaker): BCLK=14, LRC=15, DIN=26, EN=32

└─ INMP441 (small): SD=33 (share SCK/BCLK=14, WS/LRC=15)

Note: avoid changing pins 7/8 if the connection to the hub works; they are critical for the LPF2 handshake on the LMS-ESP32.

Simulator wiring screenshot (to be completed)

Add here the image with the visual wiring from the simulator:

![TriSense wiring in simulator]( ./docs/simulator-wiring.png )

Quick checklist for the picture:

- to clearly see the LMS-ESP32 board;

- to see the connections for HuskyLens, microphone and speaker;

- to see the pin labels (GPIO).

You can create the docs/ folder and put the screenshot there.

MQTT (ESP32 <-> PC)

| Topical | Direction | Content |
| --- | --- | --- |
| vision/tags | ESP32 -> broker -> PC | JSON {"id": N} (class HuskyLens: 1 = face, 2+ = LEGO objects) |
| robot/control | PC -> broker -> ESP32 | JSON commands (speak, listen, pc_host, voice_port, duration_ms) |
| robot/speak | PC -> broker -> ESP32 | Retained greeting message/short message for TTS on the robot |

Note: in main_robot.py, MQTT is non-blocking (check_msg()), and the LPF2 loop remains active (pr.process() / process_async).

Software requirements (PC)

- Python 3.10+ recommended

- Dependencies: requirements.txt

- Gemini API key in .env (GEMINI_API_KEY)

Quick installation:

pip install -r requirements.txt

copy .env.example .env

Then fill in .env:

- GEMINI_API_KEY=...

- optional: MQTT_BROKER, MQTT_PORT, MQTT_CLIENT_ID_PC

- optional voice: VOICE_TCP_PORT, TRISENSE_TTS_PC, TRISENSE_TTS_OVER_TCP

Running on PC

1) Standard mode (brain + MQTT)

py run_trisense_brain.py

Used for the main interaction flow via MQTT.

2) Voice dialog mode (small ESP -> PC STT/TTS)

py run_voice_dialog.py

This mode also starts the TCP voice server (default port 8765), which receives audio from the ESP.

ESP32 configuration

- Run the firmware from main_robot.py on the LMS-ESP32 board (from the project folder, replace COM7 as the ESP appears in Device Manager):

python -m mpremote connect COM7 run main_robot.py

- Create secrets.py on ESP (not in repo) for local keys/data (ex: GEMINI_API_KEY, PC_VOICE_IP).

- Check the Wi-Fi network and MQTT broker in the ESP configuration.

Hardware audio tests (speaker + microphone)

Before the test:

- connect the board to the correct serial port (e.g. COM7);

- stops any previous running (main_robot.py) from the same port;

- uses the same connections from the pinout (BCLK=14, LRC=15, DIN=26, MIC_SD=33).

Speaker test (MAX98357)

Script: test/test_i2s_beep.py 
Role: strictly checks the I2S TX + amplifier + speaker chain (without MQTT/HuskyLens).

Running:

python -m mpremote connect COM7 run test/test_i2s_beep.py

What you need to see/hear:

- in serial: I2S0 init OK 44100 STEREO, then write round ...;

- on speaker: repeated test tone (beep/sine).

If it is quiet:

- check the power supply of the MAX98357 module (Vin, GND);

- check DIN/BCLK/LRC and enable pin;

- test with another speaker or connect the SD/EN module directly to 3.3V (as the script mentions).

Microphone + speaker test (INMP441 + MAX98357)

Script: test/test_mic_difuzor.py 
Role: runs the speaker test and then the I2S microphone reading (serial level) in turn.

Running:

python -m mpremote connect COM7 run test/test_mic_difuzor.py

What you need to see:

- OK: I2S TX written. You should hear a tone. (speaker);

- I2S RX OK. Trying to record... (microphone);

- non-zero values for Peak L, Peak R, max when speaking into the microphone.

Quick interpretation:

- Very low signal -> problem on SD microphone / clock / power supply;

- Blocked/saturated signal -> check L/R, WS/SCK, common GND;

- Microphone seems active. -> test passed.

What each file does

The root of the project

| File | Role |
| --- | --- |
| .env.example | Example of environment variables for the PC side |
| .gitignore | Files/folders excluded from git (ex: .env, secrets.py, .venv) |
| .micropico | Configuration for uploading/running MicroPython (tooling) |
| .vscode/extensions.json | Recommended extensions for VS Code/Cursor |
| .vscode/settings.json | Editor local settings for the project |
| CONTEXT.md | Placeholder file (currently empty) |
| HARDWARE_LMS_ESP32_ANTONS.md | LMS-ESP32 hardware notes used in the paper |
| Project Report 2.0 3 Ways of Garbage (2).pdf | PDF project documentation |
| README.md | Main project documentation |
| RECOVERY.md | Recovery steps for REPL/mpremote |
| boot.py | Boot script on ESP32 |
| lpf2.py | LPF2 implementation/protocol used in LEGO communication |
| main.py | Main script for LEGO Hub (Pybricks) |
| main_robot.py | Main firmware on ESP32 (camera, MQTT, audio, hub connection) |
| child_memory.json | Local storage for conversation memory |
| pupremote.py | PUPRemote library for connecting ESP32 <-> LEGO Hub |
| pyhuskylens.py | Driver/interface for HuskyLens camera |
| rep | Local marker for REPL mode on device |
| requirements.txt | Python dependencies for the PC side |
| run_esp32.ps1 | PowerShell script to run main_robot.py via mpremote |
| run_trisense_brain.py | Entrypoint for the PC brain in standard mode |
| run_voice_dialog.py | Entrypoint for PC brain with TCP voice server |
| secrets.example.py | Example of secrets for ESP32 |
| testing/ | Folder with test scripts (audio, MQTT, laptop microphone) |
| testing/README.md | Quick guide to running all test scripts |
| test/mqtt_speak_test.py | Test utility for MQTT speak commands |
| test/mqtt_voice_listen_test.py | Test utility for MQTT listen command (voice) |
| test/test_i2s_beep.py | I2S audio test (speaker) on ESP32 |
| test/test_mic_difuzor.py | Combined microphone + speaker test on ESP32 |
| test/test_laptop_mic.py | Local laptop microphone test -> STT -> TriSense response |
| trisense_metrics.csv | Brain Running Metrics Log |

The trisense package/ simply explained

Think of trisense/ as the robot's "control room":

| File | Simple explanation |
| --- | --- |
| trisense/brain.py | It is the "boss" who coordinates everything: what the robot sees, what response it gives, when it moves on to the next step. |
| trisense/ai_client.py | Talk to the AI (Gemini) to generate friendly responses |
| trisense/config.py | "Project settings": MQTT broker, ports, topic names, files |
| trisense/mqtt_layer.py | Connects via MQTT between robot and laptop |
| trisense/memory_store.py | Remember simple information (e.g. child's name) |
| trisense/tts_engine.py | Turn text into speech on your laptop (if enabled) |
| trisense/voice_tcp_server.py | Receive audio from the robot when using voice mode |
| trisense/audio_push.py | Send audio from laptop to robot |
| trisense/metrics_logger.py | Save statistics (e.g. reaction times) |
| trisense/states.py | Defines the game states (START, GREETINGS, ACTIVITY, END) |
| trisense/__init__.py | Python package technical file |

In short: brain.py decides, ai_client.py generates the text, mqtt_layer.py sends/communicates, and the rest of the modules help with memory, voice, and statistics.

Important notes

- Do not publish API keys or Wi-Fi passwords in the repository.

- If LPF2 desynchronization or upload problems occur, see RECOVERY.md.

- For hardware and pin debugging, use HARDWARE_LMS_ESP32_ANTONS.md.